/*************************************************************/

//

// Description : LIN description file created by PREEvision

// Created     : 12/19/25 6:55 PM

//

/*************************************************************/

LIN_description_file;

LIN_protocol_version = "2.1";

LIN_language_version = "2.1";

LIN_speed = 19.2 kbps;

Nodes {

Master: ZXD, 5 ms, 0 ms;

Slaves: EBS;

}

Signals {

EBSAcumtdBatChrg_l: 16, 65535, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池累积充电量接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:UNM备注:驻车能耗&电源管理

EBSAcumtdBatDschrg_l: 16, 65535, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池累积放电量接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:UNM备注:驻车能耗&电源管理

EBSBatCrnt_l: 16, 65535, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池电流接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:UNM备注:驻车能耗&电源管理

EBSBatCrntRng_l: 2, 3, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池电流范围接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:ENM备注:驻车能耗&电源管理

EBSBatIncnstncyFlag_l: 1, 1, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池传感器一致性接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:ENM备注:驻车能耗&电源管理

EBSBatSOFVol1_l: 8, 255, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池SOF1接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:UNM备注:驻车能耗&电源管理

EBSBatSOFVol1Sts_l: 2, 3, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池SOF1有效性接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:ENM备注:驻车能耗&电源管理

EBSBatSOFVol2_l: 8, 255, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池SOF2接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:UNM备注:驻车能耗&电源管理

EBSBatSOFVol2Sts_l: 2, 3, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池SOF2有效性接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:ENM备注:驻车能耗&电源管理

EBSBatTem_l: 8, 255, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池温度接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:UNM备注:驻车能耗&电源管理

EBSBatVol_l: 14, 16383, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池电压接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:UNM备注:驻车能耗&电源管理

EBSCrntSts_l: 2, 3, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池电流有效性接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:ENM备注:驻车能耗&电源管理

EBSErInCalData_l: 1, 1, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池ECU标定数据状态接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:ENM备注:驻车能耗&电源管理

EBSErInECUIdficn_l: 1, 1, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池ECU Identification状态接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:ENM备注:驻车能耗&电源管理

EBSQucntCrnt_l: 8, 255, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池静态电流值接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:UNM备注:驻车能耗&电源管理

EBSRespEr_l: 1, 1, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池传感器响应故障接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:ENM备注:驻车能耗&电源管理

EBSSOC_l: 8, 255, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池SOC接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:UNM备注:驻车能耗&电源管理

EBSSOCSts_l: 2, 2, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池SOC有效性接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:ENM备注:驻车能耗&电源管理

EBSSOHOfCrsn_l: 8, 255, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池腐蚀状态接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:UNM备注:驻车能耗&电源管理

EBSSOHOfCrsnSts_l: 2, 3, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池腐蚀状态有效性接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:ENM备注:驻车能耗&电源管理

EBSSOHOfLAM_l: 8, 255, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池可用容量接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:UNM备注:驻车能耗&电源管理

EBSSOHOfLAMSts_l: 2, 3, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池可用容量有效性接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:ENM备注:驻车能耗&电源管理

EBSSOHOfSlphtn_l: 8, 255, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池硫化状态接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:UNM备注:驻车能耗&电源管理

EBSSOHOfSlphtnSts_l: 2, 1, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池硫化状态有效性接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:ENM备注:驻车能耗&电源管理

EBSTemSts_l: 2, 3, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池温度有效性接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:ENM备注:驻车能耗&电源管理

EBSVolSts_l: 2, 3, EBS, ZXD; // Period(ms):100.0Signal Group:功能安全:信息安全:有无服务标签:发送方描述:蓄电池电压有效性接收方描述:ZXDVCMax. acceptable latency(ms):nullRepetition Period(ms):Num. Of Repetition:Signal Type:ENM备注:驻车能耗&电源管理

}Diagnostic_signals {

MasterReqB0: 8, 0;

MasterReqB1: 8, 0;

MasterReqB2: 8, 0;

MasterReqB3: 8, 0;

MasterReqB4: 8, 0;

MasterReqB5: 8, 0;

MasterReqB6: 8, 0;

MasterReqB7: 8, 0;

SlaveRespB0: 8, 0;

SlaveRespB1: 8, 0;

SlaveRespB2: 8, 0;

SlaveRespB3: 8, 0;

SlaveRespB4: 8, 0;

SlaveRespB5: 8, 0;

SlaveRespB6: 8, 0;

SlaveRespB7: 8, 0;

}

Frames {

EBS_ICCLIN1_FrP01_ICC_LIN1: 53, EBS, 8 {

EBSBatSOFVol1Sts_l, 54;

EBSBatSOFVol1_l, 16;

EBSBatSOFVol2Sts_l, 56;

EBSBatSOFVol2_l, 24;

EBSSOCSts_l, 50;

EBSSOC_l, 0;

EBSSOHOfCrsnSts_l, 60;

EBSSOHOfCrsn_l, 40;

EBSSOHOfLAMSts_l, 58;

EBSSOHOfLAM_l, 32;

EBSSOHOfSlphtnSts_l, 52;

EBSSOHOfSlphtn_l, 8;

}

EBS_ICCLIN1_FrP00_ICC_LIN1: 52, EBS, 8 {

EBSBatCrntRng_l, 14;

EBSBatCrnt_l, 16;

EBSBatIncnstncyFlag_l, 57;

EBSBatTem_l, 32;

EBSBatVol_l, 0;

EBSCrntSts_l, 48;

EBSErInCalData_l, 54;

EBSErInECUIdficn_l, 55;

EBSQucntCrnt_l, 40;

EBSRespEr_l, 56;

EBSTemSts_l, 52;

EBSVolSts_l, 50;

}

EBS_ICCLIN1_FrP02_ICC_LIN1: 54, EBS, 4 {

EBSAcumtdBatChrg_l, 0;

EBSAcumtdBatDschrg_l, 16;

}

}

Diagnostic_frames {

MasterReq: 60 {

MasterReqB0, 0;

MasterReqB1, 8;

MasterReqB2, 16;

MasterReqB3, 24;

MasterReqB4, 32;

MasterReqB5, 40;

MasterReqB6, 48;

MasterReqB7, 56;

}

SlaveResp: 61 {

SlaveRespB0, 0;

SlaveRespB1, 8;

SlaveRespB2, 16;

SlaveRespB3, 24;

SlaveRespB4, 32;

SlaveRespB5, 40;

SlaveRespB6, 48;

SlaveRespB7, 56;

}

}

Node_attributes {

EBS {

LIN_protocol = "2.1";

configured_NAD = 0x47;

initial_NAD = 0x47;

response_error = EBSRespEr_l;

configurable_frames {

EBS_ICCLIN1_FrP00_ICC_LIN1;

EBS_ICCLIN1_FrP01_ICC_LIN1;

EBS_ICCLIN1_FrP02_ICC_LIN1;

}

}

}

Schedule_tables {

FM_Normal_ZXD_LIN3 {

EBS_ICCLIN1_FrP00_ICC_LIN1 delay 10 ms;

EBS_ICCLIN1_FrP01_ICC_LIN1 delay 10 ms;

EBS_ICCLIN1_FrP02_ICC_LIN1 delay 10 ms;

}

FM_DiagnosticsZXD_LIN3ZXD_LIN3 {

MasterReq delay 160 ms;

SlaveResp delay 160 ms;

}

}Signal_encoding_types {

EBSBatVol_l {

logical_value, 16383, "EBSBatVol_l_16383_EBSBatVol_16383_Invalid";

physical_value, 0, 16383, 9.7656E-4, 3, "";

}

EBSVolSts_l {

logical_value, 0, "EBSVolSts_l_0_No_error";

logical_value, 1, "EBSVolSts_l_1_open_circuit";

logical_value, 2, "EBSVolSts_l_2_Out_of_range_or_ADC_defect";

logical_value, 3, "EBSVolSts_l_3_LIN_Saturation_no_HW_defect";

}

EBSSOC_l {

logical_value, 255, "EBSSOC_l_255_EBSSOC_255_Invalid";

physical_value, 0, 255, 1, 0, "";

}

EBSSOCSts_l {

logical_value, 0, "EBSSOCSts_l_0_deviation_morethan_15percent";

logical_value, 1, "EBSSOCSts_l_1_deviation_lessthan_15percent";

logical_value, 2, "EBSSOCSts_l_2_deviation_lessthan_10percent";

logical_value, 3, "EBSSOCSts_l_3_IllogicalValue";

}

EBSBatCrnt_l {

logical_value, 65535, "EBSBatCrnt_l_65535_EBSBatCrnt_65535_Invalid";

physical_value, 0, 65535, 0.0078125, -256, "";

}

EBSCrntSts_l {

logical_value, 0, "EBSCrntSts_l_0_No_error";

logical_value, 1, "EBSCrntSts_l_1_open_circuit";

logical_value, 2, "EBSCrntSts_l_2_Out_of_range_or_ADC_defect";

logical_value, 3, "EBSCrntSts_l_3_3_LIN_Saturation_no_HW_defect";

}

EBSBatTem_l {

logical_value, 255, "EBSBatTem_l_255_Invalid";

physical_value, 0, 255, 1, -40, "";

}

EBSTemSts_l {

logical_value, 0, "EBSTemSts_l_0_No_error";

logical_value, 1, "EBSTemSts_l_1_not_used";

logical_value, 2, "EBSTemSts_l_2_HW_defect_ADC_Defect";

logical_value, 3, "EBSTemSts_l_3_LIN_Saturation_no_HW_defect";

}

EBSAcumtdBatChrg_l {

logical_value, 65535, "EBSAcumtdBatChrg_l_65535_Invalid";

physical_value, 0, 65535, 0.125, 0, "";

}

EBSAcumtdBatDschrg_l {

logical_value, 65535, "EBSAcumtdBatDschrg_l_65535_Invalid";

physical_value, 0, 65535, 0.125, 0, "";

}

EBSSOHOfCrsn_l {

physical_value, 0, 255, 0.0625, 0, "V";

}

EBSSOHOfCrsnSts_l {

logical_value, 0, "EBSSOHOfCrsnSts_l_0_NotLearendAtAll";

logical_value, 1, "EBSSOHOfCrsnSts_l_1_inaccurate";

logical_value, 2, "EBSSOHOfCrsnSts_l_2_accurate";

logical_value, 3, "EBSSOHOfCrsnSts_l_3_invalid";

}

EBSSOHOfLAM_l {

physical_value, 0, 255, 0.5, 0, "Ah";

}

EBSSOHOfLAMSts_l {

logical_value, 0, "EBSSOHOfLAMSts_l_0_NotLearendAtAll";

logical_value, 1, "EBSSOHOfLAMSts_l_1_LearnedAtLeastOnce";

logical_value, 3, "EBSSOHOfLAMSts_l_3_invalid";

}

EBSSOHOfSlphtn_l {

physical_value, 0, 255, 1, 0, "Percent";

}

EBSSOHOfSlphtnSts_l {

logical_value, 0, "EBSSOHOfSlphtnSts_l_0_NoLearned";

logical_value, 1, "EBSSOHOfSlphtnSts_l_1_learned";

logical_value, 3, "EBSSOHOfSlphtnSts_l_3_invalid";

}

EBSBatCrntRng_l {

logical_value, 0, "EBSBatCrntRng_l_0_current_rangefrom_minus1_to_1A";

logical_value, 1, "EBSBatCrntRng_l_1_current_rangefrom_minus200_to_200A";

logical_value, 2, "EBSBatCrntRng_l_2_current_rangefrom_minus1500_to_1500A";

logical_value, 3, "EBSBatCrntRng_l_3_Invalid";

}

EBSBatIncnstncyFlag_l {

logical_value, 0, "EBSBatIncnstncyFlag_l_0_No_Error";

logical_value, 1, "EBSBatIncnstncyFlag_l_1_Error";

}

EBSErInCalData_l {

logical_value, 0, "EBSErInCalData_l_0_No_Error";

logical_value, 1, "EBSErInCalData_l_1_Error";

}

EBSErInECUIdficn_l {

logical_value, 0, "EBSErInECUIdficn_l_0_No_Error";

logical_value, 1, "EBSErInECUIdficn_l_1_Error";

} EBSQucntCrnt_l {

logical_value, 253, "EBSQucntCrnt_l_253_Overflow";

logical_value, 254, "EBSQucntCrnt_l_254_OutOfBounds";

logical_value, 255, "EBSQucntCrnt_l_255_Invalid";

physical_value, 0, 255, 0.00390625, 0, "";

}

EBSRespEr_l {

logical_value, 0, "EBSRespEr_l_0_No_Error";

logical_value, 1, "EBSRespEr_l_1_Error";

}

EBSBatSOFVol1_l {

physical_value, 0, 255, 0.0625, 0, "Voltage";

}

EBSBatSOFVol1Sts_l {

logical_value, 0, "EBSBatSOFVol1Sts_l_0_deviation_800mv";

logical_value, 1, "EBSBatSOFVol1Sts_l_1_deviation_400mv";

logical_value, 3, "EBSBatSOFVol1Sts_l_3_illogical_Value";

}

EBSBatSOFVol2_l {

physical_value, 0, 255, 0.0625, 0, "Voltage";

}

EBSBatSOFVol2Sts_l {

logical_value, 0, "EBSBatSOFVol2Sts_l_0_deviation_800mv";

logical_value, 1, "EBSBatSOFVol2Sts_l_1_deviation_400mv";

logical_value, 3, "EBSBatSOFVol2Sts_l_3_illogical_Value";

}

}Signal_representation {

EBSAcumtdBatChrg_l: EBSAcumtdBatChrg_l;

EBSAcumtdBatDschrg_l: EBSAcumtdBatDschrg_l;

EBSBatCrntRng_l: EBSBatCrntRng_l;

EBSBatCrnt_l: EBSBatCrnt_l;

EBSBatIncnstncyFlag_l: EBSBatIncnstncyFlag_l;

EBSBatSOFVol1Sts_l: EBSBatSOFVol1Sts_l;

EBSBatSOFVol1_l: EBSBatSOFVol1_l;

EBSBatSOFVol2Sts_l: EBSBatSOFVol2Sts_l;

EBSBatSOFVol2_l: EBSBatSOFVol2_l;

EBSBatTem_l: EBSBatTem_l;

EBSBatVol_l: EBSBatVol_l;

EBSCrntSts_l: EBSCrntSts_l;

EBSErInCalData_l: EBSErInCalData_l;

EBSErInECUIdficn_l: EBSErInECUIdficn_l;

EBSQucntCrnt_l: EBSQucntCrnt_l;

EBSRespEr_l: EBSRespEr_l;

EBSSOCSts_l: EBSSOCSts_l;

EBSSOC_l: EBSSOC_l;

EBSSOHOfCrsnSts_l: EBSSOHOfCrsnSts_l;

EBSSOHOfCrsn_l: EBSSOHOfCrsn_l;

EBSSOHOfLAMSts_l: EBSSOHOfLAMSts_l;

EBSSOHOfLAM_l: EBSSOHOfLAM_l;

EBSSOHOfSlphtnSts_l: EBSSOHOfSlphtnSts_l;

EBSSOHOfSlphtn_l: EBSSOHOfSlphtn_l;

EBSTemSts_l: EBSTemSts_l;

EBSVolSts_l: EBSVolSts_l;

}